초고속 V2X 통신기반

자율주행 서비스 기술 개발

[초고속 V2X 사이드링크 Open-Loop 전력제어 성능검증서]

2023. 12. 3

| 과제번호 | 2021-0-01140 |
| --- | --- |
| 과 제 명 | 초고속 V2X 통신기반 자율주행 서비스 기술 개발 |
| 등록번호 | IHU_MTRL-2023-5GV2X_Srvc_TR01 |
| 제·개정일자 | ‘23. 12. 3 (Ver. 1.0) |

Table of Contents

- V2X Sidelink Open Loop Power Control	2

  - Open Loop Power Control in Sidelink Communications

  - Architecture of Sidelink OLPC

  - Framework of Sidelink OLPC

  - Proposed Sidelink Adaptive OLPC Algorithm

  - Simulation Scenario and Parameters Setting

  - Result Discussions

References	13

- Development of Sidelink Open Loop Power Control

  - Sidelink Open Loop Power Control

- Sidelink Open Loop Power Control is a power control technique used in wireless communication systems. This technology is primarily employed in scenarios where devices communicate directly with each other rather than through a base station.

- In such direct communication scenarios, Sidelink Open Loop Power Control helps devices autonomously adjust their transmission power without central scheduling. The term "open loop" indicates that in this process, devices adjust their power without direct feedback, as opposed to closed-loop control based on received signal quality feedback.

- Through open-loop power control, devices can autonomously adapt their transmission power based on predefined rules or guidelines to accommodate various communication conditions, such as distance and channel states. This helps improve communication reliability, reduce interference, and optimize system performance. Sidelink Open Loop Power Control is a technical means used in cellular communication systems to support direct communication between devices.

< Fig.1: Process of the OLPC and influencing factors >

< Fig.2: Process of the OLPC General Algorithm>

- Power Control System

  - Open Loop Power Control (OLPC)

  - Closed Loop Power Control (CLPC)

- Overview of the key differences

- Feedback Mechanism

  - OLPC adjusts the transmission power of a device without relying on explicit feedback from the receiver. It typically relies on predefined rules or guidelines based on factors like distance, channel conditions, and other environmental parameters.

  - CLPC, on the other hand, uses feedback from the receiver to adjust the transmission power. The receiver provides information about the quality of the received signal, and the transmitter adjusts its power accordingly to maintain a desired signal quality.

- Autonomy

  - OLPC: It operates autonomously without direct feedback. The transmitter makes power adjustments based on predetermined rules, reducing the need for continuous communication with the receiver.

  - CLPC: It requires a continuous feedback loop between the transmitter and the receiver. The transmitter adjusts power in response to real-time feedback, making it a more dynamic and responsive system.

- Complexity